- LDPC decoding engine – class ldpc_dec_engine_t

LDPC decoding engine is a class, that implements one iteration of LDPC decoder.  Interface of the class is provided in Figure 1-1.

class ldpc_dec_engine_t { public: enum class ret_status { ERROR = 0, OK = 1, ET = 2 }; ldpc_dec_engine_t(); void init(const std::vector<std::vector<int>> &check_matrix, int z); void reset(); void push(const std::vector<int> &in); ret_status iterate(); const std::vector<bool> &pull(); bool calc_parity_check(); private: ... };

Figure 1-1ldpc_dec_engine_t class interface

ldpc_dec_engine_t class member functions are described below:

ldpc_dec_engine_t() – default constructor

void init(const std::vector<std::vector<int>> &check_matrix, int z) – initialization function. Should be called before decoder engine usage. Input arguments: check_matrix – LDPC code matrix prototype as defined in Annex R in 802.11n standard. Size of the matrix is S by 24, where S is the number of subblocks in a column (4, 6, 8, 12) and 24 is the number of subblocks in a row. z – LDPC code parity check matrix subblock size, can be 27, 54, 81.

void reset() – decoder engine reset. Should be called before next codeword decoding.

void push(const std::vector<int> &in) – push sequence of LLRs to decoder. Should be called before iterate() function. Input arguments: in – sequence of LLRs. Sequence length can be 648, 1296 or 1944. LLR is a fixed point (7,6) value, where 6 is the number of fractional bits and 7 is the total number of bits. Each LLR can take values from -63 to 63.

ret_status iterate() – LDPC decoder engine iterate. Is called after a codeword was pushed to the engine. After each iteration the function returns decoding status. Return value: ret_status::ERROR in case of internal error situation; ret_status::OK decoding iteration was successful; ret_status::ET when early decoding iteration criteria was met.

const std::vector<bool> &pull() – get current decoded sequence. Returned sequence length is equal to input LLRs sequence length. After pull() function decoding iterations can be continued. Return value: true – if decoded bit is 1; false – if decoded bit is 0.

calc_parity_check() – check if decoded sequence is a valid codeword. Return value: 0 – if decoded sequence is a valid codeword; 1 – otherwise.

Other fields that are not listed here are service fields and should not be used.

- Program example

Example of ldpc_dec_engine_t class usage is provided below.

int main() { ... std::vector<std::vector<int>> check_matrix; // initialize check matrix ... int Z; // initialize Z value ldpc_dec_engine_t dec_engine; dec_engine.init(check_matrix, Z); std::vector<int> in; // initialize input LLRs const int num_iterations = 10; dec_engine.push(cw); for (int i = 0; i < num_iterations; i++) { auto ret = dec_engine.iterate(); std::cout << "iteration output: " << (int)ret << "\n"; std::cout << "final check is : " << dec_engine.calc_parity_check() << "\n"; } auto dec_cw = dec_engine.pull(); // process decoded sequence dec_engine.reset(); // process next sequence return 0; }